Recommended Environmental Performance Requirements

## IAC Revision - Version 3

Note – Version 1 & 2 track changes have been accepted and are now shown with an underline.  Change for version 3 are coloured and underlined. The changes for Version 3 reflect the Noise and Vibration evidence and changes suggested by submitters that have presented to panel including University of Melbourne, RMIT and Graduate House.

| Draft EES Evaluation Objective | EPR no. |  | Environmental Performance Requirement |  | Precinct | Timing | Risk No. | Basis for Change Version 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environmental Management Framework |  |  |  |  |  |  |  |  |
| Environmental Management Framework: To provide a transparent framework with clear accountabilities for managing environmental effects and hazards associated with construction and operation phases of the project, in order to achieve acceptable environmental outcomes | EM1 |  | - Develop a program to set out the process and timing for development of an Environmental Management System (EMS), Construction Environmental Management Plan (CEMP), Site Environment Implementation Plans (SEIP), Operations Environmental Management Plan (OEMP) and other plans as required by the Environmental Performance Requirements and as relevant to any stage of the project. The process for development of and implementation of the EMS, the CEMP the SEIP and OEMP program must include consultation with Councils, Heritage Victoria, the Roads Corporation, Melbourne Water, Public Transport Victoria, and the Environment Protection Authority and other stakeholders as relevant. These consultation processes must be described in the program. |  | All | Design/ construction/ operation | - |  |
|  | EM2 |  | Prepare and implement an Environmental Management System that is certified to ISO 14001:2015 Environmental Management Systems – requirements with guidance for use for construction and operation. Prepare a Construction Environmental Management Plan (CEMP), Site Environment Implementation Plans, Operations Environmental Management Plan (OEMP) and other plans as required by the Environmental Performance Requirements and as relevant to any stage of the project. The CEMP should be prepared in accordance with EPA Publication 480, Environmental Guidelines for Major Construction Sites (EPA 1996). |  | All | Design/ construction/ operation | - |  |
|  | EM3 |  | Appoint an Independent Environmental Auditor to undertake environmental audits of compliance with the approved CEMP, Environmental Performance Requirements and approval conditions. |  | All | Construction | - | MMRA change as all packages will have an IEA |
| Transport |  |  |  |  |  |  |  |  |
|  | New TA* |  | Traffic and Transport Working Group MMRA to establish the Traffic and Transport Working Group comprising of relevant representatives from MMRA, road management authorities, relevant councils, public transport providers and other relevant agencies as required. The Traffic and Transport Working Group will be responsible for reviewing and providing feedback on: Transport management plans Relevant designs and methodologies for monitoring implementation of Transport Management Plans Transport modelling and proposed transport network upgrades to mitigate the transport effects of constructing the project The Group should also: Invite other key affected stakeholders to present or attend where matters specific to those stakeholders in the relevant precincts are being discussed or addressed; and Advise those key affected stakeholders of potential impacts and proposed traffic and transport mitigations, and consider stakeholders’ responses on these matters in providing feedback on the transport management plan(s) required under EPR T1. |  | All | Construction |  | University of Melbourne changes 15/09/16 |
| Transport connectivity – To enable a significant increase in the capacity of the metropolitan rail network and provide multimodal connections, while adequately managing effects of the works on the broader transport network, both during and after the construction of the project | T1 |  | Road Transport (Construction Phase) - Develop and implement a transport management plan(s) in consultation with the Traffic and Transport Working Group relevant road management authorities and implement the plan(s) to minimise disruption to affected local land uses, traffic, car parking, on-road public transport, pedestrian and bicycle movements and existing public facilities during all stages of construction., including but not limited to: - The transport management plan(s) should be prepared for each precinct, and also be coordinated across the whole project to provide an overall transport management plan for the project. - The transport management plan(s) must be informed and supported by an appropriate level of transport modelling, as agreed by the Traffic and Transport Working Group, and must include, but not be limited, to: Management of any temporary or permanent full or partial closure of traffic lanes including (but not limited to): - Childers Street, Tennyson Street and Lloyd Street Kensington - Arden Street and Laurens Street, North Melbourne - Royal Parade, Grattan Street and Barry Street, Parkville - Franklin Street, A’Beckett Street and Little La Trobe Street at CBD North - Flinders Street and Flinders Lane at CBD South - Linlithgow Avenue, Melbourne - St Kilda Road, Domain Road, Albert Road at Domain - Toorak Road West at Fawkner Park (and the surrounding road network) during construction of the route 8 tram diversion along Toorak Road West between St Kilda Road and Park Street - Osborne Street, William Street in South Yarra A monitoring methodology and a program for monitoring results of the implementation of Transport Management Plans to be reported to the Traffic and Transport Working Group. If unanticipated adverse effects are further identified, mitigation measures would be developed and implemented. Monitoring of: - Travel behaviour changes caused by construction works, including pre-construction baseline data and periodic reporting on behaviour change. Use this data as an input to the design of transport networks following construction and for annual review of the transport management plan(s). - Traffic, public transport, pedestrian and bicycle movements throughout the construction period |  | All | Construction | T001 to T007 |  |
|  |  |  | Transport Traffic management plan(s) must be developed recognising other projects operating concurrently and transient businesses such as bus/walking/cycling tours and airport transfers, where relevant Provision for a minimum of one lane for traffic in each direction on St Kilda Road through the construction period within the Domain station precinct Potential routes for construction vehicles travelling to and from all Melbourne Metro construction work sites, recognising sensitive receptors and minimising the use of local streets where practicable (refer to EPR NEW NV B*) Provision of suitable routes for vehicles to maintain connectivity for road users to JJ Holland Park, South Kensington station and to the medical and educational facilities adjacent to the Parkville construction work site Provision of alternative routes for trucks accessing the 50 Lloyd Street Business Estate, Kensington Provision of alternate parking where possible to replace public parking lost from Childers Street, Laurens Street, Grattan Street, Domain Road, St Kilda Road and Albert Road during construction and preventing parking at undesignated locations on local roads |  |  |  |  | - Agreed with CoM 13/09/16 and addresses Graduate Union’s submission |
|  |  |  | Provision of car parking for construction workers where possible practicable and in this regard: Use of off-street car parks for construction workers should be by prior agreement with the relevant management body; and Measures must be implemented to prevent, to the extent practicable, construction workers parking in on-street spaces, unless it can be demonstrated by car-parking surveys that there is adequate on-street supply. |  |  |  |  | University of Melbourne changes 15/09/16 and responds in part to Citywide submission 19/09/16 |
|  |  |  | A green travel strategy to encourage construction workers to travel to / from worksites by means other than private vehicle and / or outside peak times. This should include provision for on-site tool storage where practicable. |  |  |  |  |  |
|  |  |  | Provision of suitable routes for cyclists and pedestrians to maintain connectivity and safety for roads and shared paths to provide continued access, including (but not limited to): Childers Street, JJ Holland Park, South Kensington station, Laurens Street, Grattan Street, Swanston Street, Franklin Street, Flinders Street, St Kilda Road, Albert Road, Domain Road, Toorak Road and Fawkner Park |  |  |  |  |  |
|  |  |  | - Develop and implement network enhancement projects (NEPs) in consultation with the Traffic and Transport Working Group for locations including, but not limited, to: - College Crescent, Gatehouse Street, Cemetery Road and other east-west roads in the Parkville Precinct, to accommodate traffic that may use these roads as a result of the Grattan Street closure. - Provision of complementary improvements to Kings Way, Canterbury Road and other roads and intersections to accommodate additional traffic that may use these roads and to assist traffic flow, including public transport priority treatments for affected bus and tram routes, in St Kilda Road for the duration of the works - These NEPs should have the objective of balancing impacts across the transport network and must consider the VicRoads Road Users Hierarchy principles set out in SmartRoads. |  |  |  |  | - Agreed with CoM 13/09/16 |
|  |  |  | Domain Road should be kept open from the east up to Dallas Books Drive, with provision for a local turnaround, |  |  |  |  |  |
|  |  |  | In consultation with emergency services, develop suitable measures to ensure emergency service access is not inhibited as a result of Melbourne Metro construction worksites Special arrangements for delivery or removal of large loads. |  |  |  |  |  |
|  | T2 |  | Public Transport (Construction Phase) Develop and implement a plan for occupying railway land and tracks at the western portal, eastern portal and western turnback that minimises the disruption to railway services during construction. Plan to be developed to the satisfaction of VicTrack and MTM. Provide suitable routes for pedestrians to maintain connectivity, including DDA access, for users of South Kensington Station, Melbourne Central Station, Flinders Street Station, of new tram stops proposed along Toorak Road and St Kilda Road, and around all construction sites generally In consultation with PTV, VicRoads or the relevant road management authorities, investigate and implement intersection modifications where practicable, including public transport priority treatments for affected bus and tram routes Develop and implement measures to minimise disruption to the tram and bus networks resulting from the construction of Melbourne Metro in consultation with the relevant road management authorities and to the satisfaction of PTV, including (but not limited to): Options to divert the 401, 402, 403, 505 and 546 bus services Tram routes on La Trobe Street and Swanston Street Tram routes on Flinders Street and Swanston Street Tram operations on Toorak Road and the diversion of the No. 8 tram route Periodic closures of Royal Parade tram route Tram routes on St Kilda Road Disruption to other tram routes through Domain tram stop Bus replacement services for disrupted rail customers. |  | All | Construction | T001 to T005, T008 |  |
|  | T3 |  | - Active Transport (Construction Phase) Develop and implement transport management measures in consultation with relevant authorities for cyclists and pedestrians to maintain connectivity and performance levels throughout construction for road and shared path users including (but not limited to): JJ Holland Park, South Kensington station, Laurens Street, Grattan Street, Swanston Street adjacent to Gate 4 at University of Melbourne, Franklin Street (including RMIT facilities), Swanston Street, Flinders Street, St Kilda Road, Domain Road, Domain Parklands, Albert Road, Toorak Road, Fawkner Park, Osborne Street, William Street and Chapel Street Implement active control and wayfinding information at construction work site access points to maintain safety by avoiding potential conflicts between trucks, pedestrians and cyclists In consultation with the City of Melbourne, provide suitable routes for cyclists and pedestrians throughout construction to and maintain connectivity for road and shared path users around JJ Holland Park and South Kensington station. In consultation with the City of Stonnington, provide suitable routes for cyclists and pedestrians to maintain connectivity and connection, having regard to the removal of the William Street Bridge and Lovers Walk pedestrian path during the construction phase. Provide for movement along the Tan Track in the Botanical Gardens near the Linlithgow Avenue construction sites, or provide a suitable alternative pedestrian path during construction. Maintain appropriate pedestrian access to public car parks adjacent to or within construction areas including the car park beneath University Square |  | All | Construction | T001 to T005 | - All other changes - University of Melbourne changes 15/09/16 - Dot point 3 - CoM comments 26/8/16 - Evidence of J.Sellars |
|  | T4 |  | Travel Demand Management Strategy In advance of construction works, MMRA to develop and implement a travel demand management strategy and appropriate tools to promote specific transport behaviour changes in response to road, bicycle and pedestrian paths closures/modifications and to reduce traffic congestion around construction sites, particularly in the vicinity of the Parkville and Domain precincts where road closures and restrictions are proposed. The strategy must be consistent with the MMRA Community and Stakeholder Engagement Plan and, where practicable, include a mechanism for collecting and disseminating real-time travel time information to the public. Existing traffic and public transport information channels would be used where ever possible. Engage with key stakeholders in the development, implementation and monitoring of the Travel Demand Management Strategy including, but not limited to, councils, road management authorities, public transport providers, educational facilities, research institutions, businesses, impacted community groups and other affected key stakeholders in each precinct. |  | All | Construction | T001 to T007 |  |
|  | T5 |  | Road Transport (Operational Phase) Design all roadworks and shared path works to relevant design standards to maintain safety of movement in consultation with the relevant road management authorities, as required, and be underpinned by appropriate transport modelling and consideration to minimise carpark loss to the extent practicable. Develop and implement a plan to reinstate car parking on Childers Street, Kensington and Laurens Street, North Melbourne in consultation with the relevant road management authorities that: Minimises the permanent loss of parking where possible Ensures re-instated car parking does not encroach on JJ Holland Park Considers opportunities for replacement of any net loss of parking at nearby locations Reduces the risk of overflow parking in local streets from South Kensington station and activities at JJ Holland Park Replaces loading zones to service the needs of the existing businesses in the precinct where disrupted during construction Develop and implement a plan for the reinstatement of Grattan Street, Parkville in consultation with the relevant road management authorities that includes: Optimal replacement of car parking spaces along Grattan Street to service the needs of the hospitals and the university, including the retention or replacement of specific short-term and DDA compliant parking Optimal design of the road network around Grattan Street associated with the changed demands and network changes on Grattan Street and Royal Parade/Elizabeth Street Develop and implement a plan for the future use of the Franklin Street road reserve in consultation with the relevant road management authorities that includes: Optimising the design of Franklin Street in the project area Regard to the future function of Franklin Street envisaged in the Queen Victoria Market Precinct Renewal Master Plan Monitoring the change in travel patterns around the area associated with the revised design of Franklin Street Develop and implement a plan for the design of A’Beckett street in consultation with relevant road management authorities that includes: Optimising the design of A’Beckett Street and location of station infrastructure Consideration of pedestrian and vehicle movements on Swanston Street between LaTrobe and A’Beckett Streets and on Little La Trobe Street Optimise the design of the reinstated St Kilda Road and apply the road users hierarchy in consultation with the relevant road management authorities to: Align with the conceptual design for St Kilda Road as prepared for the TAC by VicRoads, City of Melbourne and City of Port Phillip Reduce delays and congestion Maintain safe operations through the precinct Determine the optimal parking provision in the area and replace any lost parking where possible. Where vehicle and pedestrian access are altered during construction, ensure that vehicle and pedestrian access is reinstated appropriately, in accordance with relevant road design standards, so adjacent land is not compromised. |  | All | Operation | T009 to T011 | - First dot point - University of Melbourne changes 15/09/16 - Agreed with CoM 13/09/16 |
|  | T6 |  | Public Transport (Operational Phase) Review, with PTV, the bus services in the areas around Arden, Parkville, CBD North, CBD South and Domain stations including a review of the route 401 bus frequency that will have reduced demand following implementation of Melbourne Metro Optimise the design of Melbourne Metro stations to ensure integration with existing and planned future uses and so that they will provide connections: Between the new Parkville station and the new tram stop on Royal Parade For interchange between the new CBD North station and the existing tram and bus services along La Trobe Street and Swanston Street For interchange between the new CBD South station and the existing tram services along Flinders Street and Swanston Street Between the new Domain station and the new island platform trams stop in the centre of St Kilda Road and connections to the tram services along Domain Road Implement measures to address pedestrian congestion at and around station entrances where they interface with the Precincts, to the extent practicable Provide adequate wayfinding to facilitate passenger transfers (Refer to EPR LU4) Review, with PTV and Yarra Trams, the bus and tram services in the area to optimise the functionality of the CBD North and South stations and to reduce the reliance on the Swanston Street tram corridor. |  | All | Operation | T009 to T010 | Agreed with CoM 13/09/16 |
|  | T7 |  | Active Transport (Operational phase) Develop and implement a permanent pedestrian footpath and on-road bicycle design for Childers Street, Kensington with the relevant road management authority, local council, and the land manager prior to the removal of the shared use path on the southern side of the street In cooperation with the relevant road management authority and local council, and where practicable to do so, re-instate on-road bicycle lanes and bicycle parking provisions removed during construction Provide appropriate bicycle parking at each station adopting a flexible design that would allow for future expansion of capacity, if required. Review the reinstatement and provision of safe and effective bicycle lanes in and around the Melbourne Metro station sites in cooperation with the road authority and the local council Provide wayfinding information to enhance connectivity for pedestrians and public transport users including (but not limited to) the following locations: Between Melbourne Central Station and the new CBD North Station The underground connection between Flinders Street Station and the new CBD South Station. At modal interchanges between new Melbourne Metro stations and other transport modes |  | All | Operation | T009 to T010 | Agreed with CoM 13/09/16 Response in part to COPP submission |
|  | New TB* |  | - Waste collection - Develop and implement a plan in consultation with local councils and private waste collection services to manage changes to waste collection and waste storage in the construction area. The plans should include, but not be limited to: Providing access for existing waste collection services from existing businesses and residential properties considering the extent of the construction areas and road network changes, Providing access to alternative waste collection locations for businesses during project construction and operation where existing waste disposal location are removed Design for re-instatement of appropriate access for existing waste services during project operation Consultation with affected businesses, land owners and residents to be undertaken jointly with local councils to encourage alternative waste management options to be adopted. |  | All | Design and construction |  |  |
|  |  |  | - Also refer to the following Environmental Performance Requirements for ‘Transport’: New NVB* |  |  |  |  |  |
| Greenhouse Gas |  |  |  |  |  |  |  |  |
| Transport connectivity – To enable a significant increase in the capacity of the metropolitan rail network and provide multimodal connections, while adequately managing effects of the works on the broader transport network, both during and after the construction of the project | G1 |  | Develop and implement a Sustainability Management Plan to meet, as a minimum, the Melbourne Metro sustainability targets, including achieving the specified ratings under the Infrastructure Sustainability Council of Australia’s Infrastructure Sustainability Rating Tool and the Green Star Design and As Built Melbourne Metro Rail Tool. |  | All | Design/ Construction/ Operation | GH001, GH002 |  |
|  | G2 |  | Monitor and report on how each of the best practice GHG abatement measures and sustainability initiatives identified in the Concept Design is implemented in the detailed design of the project and whether any additional measures not included in the Concept Design are feasible. |  | All | Design | GH001, GH002 |  |
| Land Use and Planning |  |  |  |  |  |  |  |  |
| Built environment – To protect and enhance the character, form and function of the public realm and buildings within and adjacent to the project alignment, and particularly in the vicinity of project surface structures, having regard to the existing and evolving urban context | LU1 |  | - Develop and implement a plan for construction and operation of Melbourne Metro that has the purpose of minimising impacts to the development and/or operation of existing land uses, including: Limiting the permanent change of use within existing public open space Minimising footprints of construction sites and permanent infrastructure on public land The location and design of all project works to avoid, to the extent practicable, temporary and permanent loss of public open space and be designed to maximise the re-instatement potential. Minimising impacts to existing public open spaces and recreational facilities and the users of these facilities, including (but not limited to): JJ Holland Park, University Square, the Melbourne City Baths, City Square, Federation Square, the Shrine of Remembrance and the Shrine Reserve, Domain Parklands, Edmund Herring Oval, and the Albert Road Reserve. Minimising the impacts to existing residential areas by locating new above ground infrastructure, such as electrical substations in appropriate locations considering adjoining properties and exploring the co-location of rail infrastructure facilities where practicable. - Such measures must be developed in consultation with affected land managers for public land. |  | All | Construction/ Operation | LU001 to LU008 | - Agreed with CoM 13/09/16 |
|  | LU2 |  | Development of the project is to be generally in accordance with the relevant Open Space Master Plans (including but not limited to, the Domain Parklands, and University Square Master Plans) in designing and constructing above-ground infrastructure for the tunnels. Consultation must occur with land managers and/or agencies responsible for the implementation of the relevant Open Space Master Plans. |  | All | Design /Construction | LU002, LU005, LU006 | Agreed with CoM 13/09/16 |
|  | LU3 |  | - Develop and implement a plan for the design and construction of Arden station that adopts an integrated approach to urban design and planning of the station and which is generally in accordance with consider the Vision and Framework Plan for Arden. ongoing strategic planning of the Arden-Macaulay Urban Renewal Area and This must include consultation with the Metropolitan Victorian Planning Authority, City of Melbourne and any other relevant agencies such as Melbourne Water. The design must include integrated water sensitive urban design (EPR SW2) and management of the extent of flooding across the site. |  | 3 – Arden station | Design/ Construction | LU007, LU008 | Agreed with CoM 13/09/16 |
|  | LU4 |  | Prior to the development of the detailed design of all permanent structures, prepare and implement strategies in accordance with Develop and implement a plan to ensure the design meets the Melbourne Metro Urban Design Strategy and relevant planning schemes that considers: Permanent above ground structures Temporary structures adopting principles of the Growing Green Guide 2014 including green walls, roofs and facades, where practicable the MMRA Creative Strategy Community identity and cultural strategy Wayfinding, signage and advertising for above ground elements of the project - The strategies must be developed in consultation with relevant local councils and land managers. - (See Environmental Performance Requirement LV1) |  | All | Design |  |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Land Use Planning’: B1, SC2, SC4, LV1, LV2, SW2 |  |  |  |  |  |  |  |
| Business |  |  |  |  |  |  |  |  |
| Social, community, land use and business: To manage effects on the social fabric of the community in the area of the project, including with regard to land use changes, community cohesion, business functionality and access to services and facilities, especially during the construction phase | B1 |  | Reduce the disruption to businesses from direct acquisition or temporary occupation of land, and work with business and land owners to endeavour to reach agreement on the terms for possession of the land. |  | All | Design | B006 |  |
|  | B2 |  | Prepare a business disruption plan consistent with a Community and Stakeholder Engagement Management Plan (SC3) to manage potential impacts to non-acquired businesses, commercial property owners and to engage with local councils, businesses, property owners and the community throughout construction. The plan shall outline the stakeholder engagement measures for each precinct and shall include: Timely information on key project milestones Changes to traffic and parking conditions and duration of impact A project construction schedule developed in coordination with transport authorities and local councils and in consultation with businesses to minimise cumulative impacts of this and other projects Plans for notifying customers of proposed changes to business operations, including the setting of suitable timeframes for notification prior to commencement of works Measures to ensure access to businesses is maintained for customers, delivery and waste removal unless there has been prior engagement with affected businesses (including mutually agreed mitigation measures as required). This could include the installation of directional and business signage to assist customers and agreed protocols for engaging with service providers (i.e. deliveries, collections, etc) Process for registering, management and resolution of complaints from affected businesses consistent with Australian Standard AS/NSZ 10002:2014 Guidelines for Complaint Management in Organisations. Measures for supporting affected businesses during construction in accordance with the Business Support Guidelines for Construction such as marketing and promotion, local activation, way-finding programs and upskilling opportunities. |  | All | Construction | B001, B002, B004, B005, B007, B008, B009 | Title of the plan has been changed to align with the contractual requirements. Agreed with CoM 13/09/16 |
|  | B3 |  | Following consultation with potentially affected businesses and prior to main works and shaft construction commencing, prepare management plans and during construction implement those plans to minimise dust, noise and vibration impacts during construction, as per AQ1, NV1 and NV4. |  | All | Construction | B003 | - University of Melbourne changes 15/09/16 |
|  | B4 |  | Maintain vehicular and pedestrian access to hospital emergency departments at all times during construction and to other key health and medical facilities, where practicable. |  | All | Construction | B001, B004 |  |
|  | B5 |  | Develop a stop work contingency plan for Class 1 emergencies (as defined in the Emergency Management Act 2013) in consultation with medical institutions in the Parkville precinct in the event that Melbourne Metro construction works are required to cease. |  | 4 – Parkville station | Design/ Construction | B001, B004 |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Business’: T1, SC2, SC3, LU1, AQ1, NV1 |  |  |  |  |  |  |  |
| Social and Community |  |  |  |  |  |  |  |  |
| Social, community, land use and business: To manage effects on the social fabric of the community in the area of the project, including with regard to land use changes, community cohesion, business functionality and access to services and facilities, especially during the construction phase | SC1 |  |  | Reduce the disruption to residences from direct acquisition or temporary occupation through measures such as: Using a case-management approach for all project interactions with affected landowners Appointing a social worker, buyers’ advocate or equivalent to assist households with special needs manage the transition Taking into account relative vulnerability and special needs of occupants Purchasing properties early when supported by the landowner. | All | Pre-construction | SC028, SC029, SC030, SC031 |  |
|  | SC2 |  |  | Prior to main works and shaft construction in areas affected, develop a relocation management framework that responds to the Residential Impact Mitigation Guidelines to ensure a uniform approach across the project for the voluntary (temporary) relocation of households subject to: Construction activities likely to unduly affect their amenity (e.g. out of hours works or sustained loss of amenity during the day for shift workers) Loss of access. | All | Pre-construction | SC001, SC004, SC005, SC006, SC009 |  |
|  | SC3 |  |  | Prior to main works and shaft construction, develop and implement a Community and Stakeholder Engagement Management Plan to engage potentially affected stakeholders and advise them of the planned construction activities, project progress, mitigation measures and intended reinstatement measures where applicable. This plan should integrate all project activities that potentially impact on community and business operations and provide for a well-coordinated communication and engagement process. The plan must include: Measures to minimise impacts to the development and/or operation of existing facilities Measures for providing advance notice of significant milestones, changed traffic conditions, changed access and parking conditions, periods of predicted high noise and vibration activities Measures for communicating the design and results from environmental monitoring programs (e.g. vibration, noise, dust, ground movement). Process for registering, d managing and resolving complaints consistent with Australian Standard AS/NSZ 10002:2014 Guidelines for Complaint Management in Organisations. Measures to address any other matters which are of concern to potentially affected stakeholders through the construction of the project. The plan must consider each precinct and station location in detail. Stakeholders to be consulted relevant to each precinct and considered in the plan include (but are not limited to): Local councils Land managers Potentially affected residents Potentially affected businesses Recreation, sporting and community groups and facilities Royal Melbourne Hospital, Victorian Comprehensive Cancer Centre, Peter Doherty Institute and other health and medical facilities The University of Melbourne RMIT Melbourne Grammar School Fawkner Park Children’s Centre and Kindergarten South Yarra Senior Citizens Centre Other public facilities in proximity. | All | Pre-construction | SC001 to SC023, SC027, SC032 to SC038, SC041, SC043, SC044, SC045 | Agreed with CoM 13/09/16 - University of Melbourne changes 15/09/16 |
|  | SC4 |  |  | Prior to main works and shaft construction commencing, work with the City of Melbourne to identify and implement suitable possible alternative areas of for use as an alternate public open space, incorporating vegetation, for community use during the construction phase to minimise the impacts of loss of the existing public open space in City Square that is are to be utilised as a construction worksites. | All6 - CBD South | Pre-construction | SC007, SC019, SC024, SC034 |  |
|  | SC5 |  |  | - Work with relevant local councils to plan for and coordinate with key stakeholders during major public events. This should include, but not be limited to: Timely provision of construction schedules to allow for appropriate event planning Timely notification of schedule changes that may impact upon major public events Consideration of appropriate alternative sites and routes for events and parades | All | Construction | SC007, SC013, SC033 |  |
|  | SC6 |  |  | In consultation with the City of Melbourne, develop a relocation strategy for sports clubs and other formal users of directly impacted recreational facilities. This strategy should aim, where available, to identify available local alternative facilities for formal recreational users displaced from recreational facilities by the project. This strategy should avoid displacing existing users at alternative facilities and provide adequate notification to clubs to minimise the impact of relocation. | All | Construction | SC024, SC025 |  |
|  | SC7 |  |  | In consultation with key stakeholders and in accordance with the Melbourne Metro Urban Design Strategy, relevant statutory approvals and other relevant requirements: a) improve community access to open or recreational space within the CBD by identifying potential opportunities to return as much land as possible used for construction to permanent public open space at City Square and Federation Square; b) re-establish sites impacted by construction works to be generally in accordance with open space master plans, including (but not limited to): - Childers Street, Kensington - JJ Holland Park - Royal Parade and Grattan Street, Parkville - City Square - Federation Square - The south western entrance of the proposed CBD South station - St Kilda Road boulevard - Edmund Herring Oval - Fawkner Park and Fawkner Park Tennis Facility - Osborne Street Reserve - South Yarra Siding Reserve - Lovers Walk - The South African Soldiers Memorial. - (See Environmental Performance Requirement LV2 and LU2.) | All | Design/ Construction | SC014, SC033, SC035, SC039, SC040, SC041 | Point a) combines SC8 with SC7 as suggested by IAC |
|  |  |  |  |  |  |  |  |  |  | Combined with SC8 as suggested by IAC |
|  | SC9 |  |  | In consultation with the City of Melbourne, develop a plan to utilise part of the Franklin Street road reserve for public open space post-construction. Plans must be in accordance with the Melbourne Metro Urban Design Strategy. | 5 – CBD North station | Design/ Construction | SC014 |  |
|  | SC New A* |  |  | In consultation with local councils and relevant state government agencies, develop and implement a plan for managing the potential impacts on vulnerable and homeless people that could be affected due to construction activity. | 5 – CBD North station and 6 – CBD South station | Design/ Construction |  |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Social and Community’: T1, T3, T4, LU1, AR2, LV1, LV2, LV3, New NVB* |  |  |  |  |  |  |  |
| Air Quality |  |  |  |  |  |  |  |  |
| Amenity: To minimise adverse air quality, noise or vibration effects on the amenity of nearby residents and local communities, as far as practicable, especially during the construction phase | AQ1 |  | Develop and implement plan(s) for dust management and monitoring, in consultation with EPA and the owners of key sensitive receptors, to minimise and monitor the impact of construction dust and advise the community of the plan. The plan must: - Set out air quality criteria and outline the justification for those criteria for above ground construction works - Be informed by air modelling of construction activities, which should identify the main dust sources and sensitive land uses - Describe the proposed air quality management system including (but not necessarily limited to): - Routinely reviewing weather model predictions - Continuous monitoring and real-time alert systems in the event of measured exceedances - Protocols for record-keeping - Protocols to ensure that site personnel advise the site manager if excessive dust emissions are observed - Describe the measures that would be implemented to ensure compliance with air quality criteria. Address monitoring requirements for key sensitive receptors, including (but not limited) to: Residential and commercial properties, including ACMI Hospitals and research facilities within the Parkville precinct Heritage listed places sensitive to dust including St Pauls Cathedral and the Melbourne City Baths Universities, including The University of Melbourne and RMIT Schools, including Melbourne Grammar School (Wadhurst Campus) and Christ Church Grammar School The Arts Centre Melbourne and National Gallery of Victoria Public parks including the Shrine of Remembrance Reserve and JJ Holland Reserve. Undertake air modelling for construction to inform the dust management plan. |  | All | Construction | AQ001 to AQ010 | - First sentence - University of Melbourne changes 15/09/16 and Graduate Union changes in part - Evidence of Bellair. - Joint witness statement Item 2 and evidence of Bellair - Lakmaker Evidence (in agreement with Bellair at [5.4]. - Submission of NGV and Arts Centre |
|  | AQ2 |  | Manage construction activities to minimise dust and other emissions in accordance with EPA Publication 480, Environmental Guidelines for Major Construction Sites (EPA 1996). |  | All | Construction | AQ001 to AQ010 |  |
|  | AQ3 |  | Control the emission of smoke, dust, fumes and other pollution into the atmosphere during construction and operation in accordance with the SEPPs for Air Quality Management and Ambient Air Quality. |  | All | Construction/ Operation | AQ001 to AQ010 |  |
| Noise & Vibration |  |  |  |  |  |  |  |  |
| Amenity: To minimise adverse air quality, noise or vibration effects on the amenity of nearby residents and local communities, as far as practicable, especially during the construction phase | NV1 |  | Manage construction noise in accordance with EPA Publication 1254 Noise Control Guidelines or as otherwise specified in the Construction Noise and Vibration Management Plan prepared under NVB*. |  | All | Construction | NV001 | Joint witness statement Items 6 and 10, and consistent with evidence of Mr Fearnside and Mr Leo |
|  | NV2 |  | For construction works conducted between CBD South station and Domain station, comply with the requirements of the Notification of Referral Decision for the Melbourne Metro Rail Project (EPBC 2015/7549, dated 22 September 2015) under the EPBC Act for vibration monitoring and measurement, as follows: Conduct pre-construction dilapidation surveys of the nearest Commonwealth Heritage listed structures to the construction activity, including the Former Guardhouse (Block B), to record structural condition and structural integrity prior to commencement of tunnelling Conduct vibration monitoring at the commencement of tunnelling in geological conditions that are similar to those at Victoria Barracks in order to quantify the actual tunnel boring machine vibration characteristics (level and frequency) for comparison to the values derived from the literature and the German DIN (DIN 4150) target Conduct continuous vibration monitoring at the nearest Victoria Barracks heritage structures to the construction activity, including the Former Guardhouse (B Block), to assess the actual tunnelling vibration for acceptability, taking into account both the vibration frequency and condition of structures, until monitoring of vibration at the Former Guardhouse (B Block) shows measurements equivalent to preconstruction vibration readings at the Former Guardhouse (B Block) If monitoring conducted according to the above demonstrates the condition of heritage structures may be degraded as a result of vibration, ground vibration must be reduced by adjusting the advance rate of the tunnel boring machine until monitoring of vibration at the Former Guardhouse (B Block) shows consistent measurements equivalent to preconstruction vibration readings at the Former Guardhouse (B Block). - (See Environmental Performance Requirement New CHA) |  | 1 – Tunnels (between CBD South station and Domain station) | Construction | NV002, NV014 | Joint witness statement Items 7, and consistent with the evidence of Mr Lovell |
|  | NV3 |  | Appoint a suitably qualified acoustic and vibration consultant to predict construction noise and vibration (through modelling) and update the modelling to reflect current construction methodology, site conditions and specific equipment noise and vibration levels (this will require noise and vibration measurements). The model would be used to determine appropriate mitigation to achieve the Environmental Performance Requirements. The acoustic and vibration consultant will also be required to undertake noise and vibration monitoring to assess levels with respect to Guideline Targets specified in the Environmental Performance Requirements. Where monitoring indicates exceedances of Guideline Targets, apply appropriate management measures as a soon as possible. The acoustic and vibration consultant will document the modelling and mitigation investigation in a Construction Noise and Vibration Assessment Report for review by the Independent Environmental Auditor, which shall provide the basis for the development of the construction noise and vibration management plan required under EPR NVB*. |  | All | Construction | NV001 to NV029 | Joint expert witness statement Item 8 |
|  | NV4 |  | Prepare and implement a communications plan to liaise with potentially affected community stakeholders and land owners regarding potential noise and vibration impacts. The plan shall include procedures for complaint management. In developing the plan, consult with relevant councils, EPA Victoria, the Parkville Precinct Reference Group and RMIT. |  | All | Construction | NV001 to NV029 | University of Melbourne changes 15/09/16 |
|  |  |  | NV5 |  | Airborne Construction Noise Guideline Targets (Internal) Implement management actions if construction noise is predicted to exceed the internal noise levels below for Highly Sensitive Areas (based on AS/NZS 2107:2000) and a noise sensitive receptor is adversely impacted. Highly Sensitive Area; Maximum Internal Construction Noise Level LAeq, 15 mins. Intensive Care Wards; 45. Operating Theatres; 45. Surgeries; 45. Wards; 40. Teaching Spaces; 45. - If construction exceeds the internal noise levels above: Consider the duration of construction noise Consider the existing ambient noise levels Consult with the owner or operator of the noise sensitive receptor Consider any specific acoustic requirements of specialist space - to determine whether a noise sensitive receptor is adversely impacted and whether management actions are required.. - (See Environmental Performance Requirement New NVB*(subclause 3)) | All | Construction | NV001 | Joint witness statement Item 10 and University of Melbourne changes 15/09/16 |
|  | NV6 |  | Vibration Guideline Targets for Structures Implement management actions if, due to construction activity, the following DIN 4150 Guideline Targets for structural damage to buildings (for short-term vibration or long-term vibration) are not achieved. Short-term vibration on structures Type of structure; Vibration at the foundation, mm/s (Peak Component Particle Velocity); Vibration at horizontal plane of highest floor at all frequencies. 1 to 10 Hz; 10 to 50 Hz; 50 to 100 Hz1; mm/s (Peak Component Particle Velocity). Type 1: Buildings used for commercial purposes, industrial buildings and buildings of similar design; 20; 20 to 40; 40 to 50; 40. Type 2: Dwellings and buildings of similar design and/or occupancy; 5; 5 to15; 15 to 20; 15. Type 3: Structures that have a particular sensitivity to vibration e.g. heritage buildings; 3; 3 to 8; 8 to 10; 8. Notes 1 It may be necessary, in accordance with New NVB*(6), to modify the guidelines targets for particular structures following the completion of pre-construction condition surveys. 2 At frequencies above 100 Hz, the values given in this column may be used as minimum values. 3 Vibration levels marginally exceeding those vibration levels in the table would not necessarily mean that damage would occur and further investigation would be required to determine if higher vibration levels can be accommodated without risk of damage. 4 For civil engineering structures (e.g. with reinforced concrete constructions used as abutments or foundation pads) the values for Type 1 buildings may be increased by a factor of 2. 5 Short-term vibration is defined as vibration which does not occur often enough to cause structural fatigue and which does not produce resonance in the structure being evaluated. | All | Construction | NV002, NV003, NV014, NV015, NV016, NV017 | Joint witness statement Item 11 |
|  |  |  | Long-term vibration on structures Type of Structure; Vibration Velocity, mm/s (Peak Component Particle Velocity) in horizontal plane at all frequencies. Buildings used for commercial purposes, industrial buildings and similar design; 10. Dwellings and buildings of similar design and/or occupancy; 5. Structures that have a particular sensitivity to vibration, e.g. heritage buildings; 2.5. Notes 1 It may be necessary, in accordance with New NVB*(6), to modify the guidelines targets for particular structures following the completion of pre-construction condition surveys. 2 Vibration levels marginally exceeding those in the table would not necessarily mean that damage would occur and further investigation is required would be required to determine if higher vibration levels can be accommodated without risk of damage. 3 Long-term vibration means vibration events that may result in a resonant structural response. |  |  |  | Joint witness statement Item 11 |
|  | NV7 |  | Vibration Guideline Targets for Above-ground Utility Assets and Infrastructure Undertake condition assessments of above-ground utility assets and infrastructure, including (but not limited to) the Arden Street Bridge and Princess Bridge, to establish construction vibration limits with asset owners. Monitor vibration during construction to demonstrate compliance with the relevant vibration guideline targets under NV6. Take remedial action if limits are not met. (See Environmental Performance Requirement New CHA*) |  | All | Construction | NV002, NV003, NV014, NV015, NV016, NV017 | Joint witness statement Item 12 |
|  | NV8 |  | Vibration Guideline Targets for Below-ground Infrastructure Undertake condition assessments of below-ground infrastructure, including (but not limited to) Swanston Street Brick Drain and Flinders Street Drain, to establish construction vibration targets with the asset owner. - Implement management actions if agreed construction vibration targets or if no specific targets have been established the DIN 4150 Guideline Targets, for buried pipework/underground infrastructure from construction are not achieved. Pipe material; Vibration Velocity, mm/s (PPV). Steel; 100. Clay, concrete, reinforced concrete, prestressed concrete, metal; 80. Masonry, plastic; 50. Notes 1 These values may be reduced by 50% when evaluating the effects of long-term vibration on buried pipework. 2 It is assumed pipes have been manufactured and laid using current technology (however it is noted that this is not the case for the majority of buried pipework potentially affected by Melbourne Metro). 3 Compliance with asset owner’s Utility Standards is to be achieved. | All | Construction | NV002, NV003, NV014, NV015, NV016, NV017 | Joint witness statement Item 12 |
|  | NV9 |  | - Vibration Dose Values (VDVs) (Human Comfort) - Implement management actions if the following Guideline Targets (VDVs) (based Table 1 in BS6472-1:2008) for continuous (as for TBMs and road headers), intermittent, or impulsive vibration are not achieved. - Location; - VDV (m/s1.75). - Day 7:00am to 10:00pm; - Night 10:00pm to 7:00am. - Preferred Value; - Maximum Value; - Preferred Value; - Maximum Value. - Residences; - 0.20; - 0.40; - 0.10; - 0.20. - Offices, schools, educational institutions, places of worship; - 0.40; - 0.80; - 0.40; - 0.80. - Workshops; - 0.80; - 1.60; - 0.80; - 1.60. - Notes 1 The Guideline Targets are non-mandatory; they are goals that should be sought to be achieved through the application of feasible and reasonable mitigation measures. If exceeded then management actions would be required. 2 The VDVs may be converted to PPVs within a future noise and vibration construction management plan | All | Construction | NV004, NV005, NV006, NV018, NV019, NV020, NV021 |  |
|  | NV10 |  | Sensitive Equipment Guideline Targets Implement management actions if equipment manufacturer specifications or measured background levels (whichever are higher) are expected to be exceeded for vibration sensitive equipment during construction at the Parkville and CBD North precincts. Where equipment manufacturer specifications are not available for vibration, adopt the applicable ASHRAE Equipment Vibration Guideline Targets: Equipment requirements; Curve. Bench microscopes up to 100x magnification; laboratory robots; Operating Room. Bench microscopes up to 400x magnification; optical and other precision balances; co-ordinate measuring machines; metrology laboratories; optical comparators; micro electronics manufacturing equipment; proximity and projection aligners, etc; VC-A. Microsurgery, eye surgery, neurosurgery; bench microscope at magnification greater than 400x; optical equipment on isolation tables; microelectronic manufacturing equipment such as inspection and lithography equipment (including steppers) to 3mm line widths; VC-B. Electron microscopes up to 30,000x magnification; microtomes; magnetic resonance images; microelectronics manufacturing equipment such as lithography and inspection equipment to 1mm detail size; VC-C. Electron microscopes at magnification greater than 30,000x; mass spectrometers; cell implant equipment; microelectronics manufacturing equipment such as aligners, steppers and other critical equipment for phot-lithography with line widths of ½ micro m; includes electron beam systems; VC-D. Unisolated laser and optical research systems; microelectronics manufacturing equipment such as aligners, steppers and other critical equipment for photolithography with line widths of ¼ micro m; includes electron beam systems; VC-E. - Notes 1 Background vibration and noise must be measured in accordance with equipment environmental test requirements. - 2 Monitoring must be undertaken in accordance with equipment specifications to demonstrate compliance, and monitoring locations be determined in consultation with operators of sensitive equipment (See Environmental Performance Requirement New NVB*(19 iv)). | 4 – Parkville station 5 – CBD North station | Design/ Construction/ Operation | NV008, NV009 | Joint witness statement Item 15 and University of Melbourne Changes 15/9/16 |
|  | NV11 |  | Ground-borne (internal) Noise Guideline Targets for Amenity Implement management actions as determined in consultation with potentially affected land owners to protect amenity at residences, sleeping areas in hospital wards, student accommodation and hotel rooms where the following ground-borne noise Guideline Targets (from the NSW Interim Construction Noise Guideline) are exceeded during construction. Implement management actions, as determined in consultation with potentially affected land owners, where ground-borne noise levels unreasonably limit usage in educational institutions such as lecture theatres. Time Period; Internal LAeq,15min, dB. Evening, 6pm to 10pm; 40. Night, 10pm to 7am; 35. Notes 1 Levels are only applicable when ground-borne noise levels are higher than airborne noise levels. 2 The noise levels are assessed at the centre of the most affected habitable room. 3 Management actions include extensive community consultation to determine acceptable level of disruption and provision of respite accommodation in some circumstances. | All | Construction | NV025, NV026, NV027, NV028, NV029 | Joint witness statement Item 16 and University of Melbourne Changes 15/9/16 |
|  | NV12 |  | - Blasting - Comply with Australian Standard AS2187.2-2006, Explosives – Storage and use Part 2 – Use of explosives for all blasting - For intensive care wards, hospital wards, operating theatres, surgeries and Bio-resources and areas with vibration-sensitive equipment which are not covered in AS2187.2-2006, develop a plan in consultation with facilities owners that: - Avoids damage to vibration-sensitive equipment Minimises adverse impact on Highly Sensitive Areas and limit adverse impacts on Bio-resources. |  | 4 – Parkville station | Construction | NV017, NV019 | University of Melbourne Changes 15/9/16 |
|  | NV13 |  | Bio-Resources and Sensitive Research Implement management actions where the following guideline targets are expected to be exceeded for areas housing bio-resources: Background noise should be kept below 50 dB and should be free of distinct tones (internal) Short exposure should be kept to less than 85 dB (internal). Notes 1 The levels above should take into consideration the frequency threshold for the Bio-resource under consideration. 2 Higher levels may be acceptable if it can be shown that the Bio-resource under consideration is exposed to higher levels and is not adversely impacted by them. 3. Noise includes airborne and ground-borne born noise at the sensitive receptors. 4. Consider the existing ambient noise levels when assessing predicted exceedances. 5. During the construction phase, a continuous monitoring program shall be implemented in accordance with EPR NVB 19(iv). |  | 4 – Parkville station 5 – CBD North station | Construction/ operation | NV012 | Joint witness statement Item 18 and University of Melbourne Changes 15/9/16 |
|  | New NVA* |  | Establish a Parkville Reference Group comprising of relevant government agencies including MMRA, PTV, VicRoads, the Victorian Department of Health and Human Services, Ambulance Victoria, Yarra Trams, and key institutions in the Parkville Precinct. |  | 4 – Parkville | Construction |  |  |
|  | NV14 |  | Appoint a suitably qualified acoustic and vibration consultant to predict noise and vibration and determine appropriate mitigation to achieve the Environmental Performance Requirements. The acoustic and vibration consultant would also be required to undertake commissioning noise and vibration measurements to assess levels with respect to the Environmental Performance Requirements. The acoustic and vibration consultant shall prepare an Operation Noise and Vibration Report for review by the Independent Environmental Auditor, which documents the predictions and mitigation measures during commissioning. |  | All | Operation | NV030 to NV038 | Joint witness statement Item 19 and University of Melbourne Changes 15/9/16 |
|  | NV15 |  | Victorian Passenger Rail Infrastructure Noise Policy (PRINP) Avoid, minimise or mitigate rail noise where the following PRINP (April 2013) Investigation Thresholds are exceeded during operation: Time; Type of Receiver; Investigation Thresholds. Day (6am – 10pm); Residential dwellings and other buildings where people sleep including aged persons homes, hospitals, motels and caravan parks Noise sensitive community buildings, including schools, kindergartens, libraries; 65 dBLAeq and a change in 3 dB(A) or more or 85 dBLAmax and a change in 3 dB(A) or more. Night (10pm – 6am); Residential dwellings and other buildings where people sleep including aged persons homes, hospitals, motels and caravan parks; 60 dBLAeq and a change in 3 dB(A) or more or 85 dBLAmax and a change in 3 dB(A) or more. Notes 1 If an investigation shows that the thresholds are not exceeded, then no further action is considered under the PRINP. 2 LAmax, is defined as maximum A-weighted sound pressure level and is the 95 percentile of the highest value of the A-weighed sound pressure level reached within the day or night. 3 For Melbourne Metro the location of assessment is at 1m from the centre of the window of the most exposed external façade. | All | Operation | NV031 |  |
|  | NV16 |  | Noise from Fixed Plant For operation, noise from fixed plant associated with Melbourne Metro shall: - Comply with State Environment Protection Policy (Control of Noise from Commerce, Industry and Trade) No. N-1 (SEPP N-1). - Where SEPP N-1 does not apply, comply with the Satisfactory Recommended Design Sound Levels as defined in AS/NZS 2107 for the following sensitive uses: - Teaching spaces - Laboratories - Conference rooms - Libraries - Music studios - Operating Theatres / Surgeries - Wards / Recliners - Performance spaces / Galleries - Places of worship If the existing background noise level within any of the above spaces exceeds the Maximum Recommended Design Sound Level in AS/NZS 2107, then noise from the fixed plant associated with the Melbourne Metro project shall not exceed the existing background levels within these spaces at the commencement of operation. This does not apply to trains and trams. |  | All | Design/ Operation | NV032, NV035 | Joint witness statement item 20 and University of Melbourne Changes 15/9/16 and RMIT suggestion |
|  | NV17 |  | Ground-borne Noise Guideline Targets for Operation Where operational ground-borne noise trigger levels are predicted to be exceeded for sensitive occupancies as shown in the table below (trigger levels are based on the Rail Infrastructure Noise Guideline, 17 May 2013 (RING(1)), assess feasible and reasonable mitigation to reduce noise towards the relevant ground-borne noise trigger level. Sensitive land use; Time of day; Internal noise trigger levels. Residential; Day (7am-10pm); 40 dBLASmax and an increase in existing rail noise level by 3 dB(A) or more. Night (10pm-7am); 35 dBLASmax and an increase in existing rail noise level by 3 dB(A) or more. Schools, educational institutions, places of worship; When in use; 40-45 dBLASmax and an increase in existing rail noise level by 3 dB(A) or more. Hospitals(bed wards and operating theatres); 24 hours; 35 dB(A) LASMax. Offices; When in use; 45 dB(A) LASMax. Cinemas and Public Halls; When in use; 30 dB(A) LASMax. Drama Theatres; When in use; 25 dB(A) LASMax. Concert halls, Television and Sound Recording Studios; When in use; 25 dB(A) LASMax. Notes 1 RING provides trigger levels for residential and schools, educational institutions and places of worship, but does not provide guidance on acceptable ground-borne noise levels for other types of sensitive receivers. Ground-borne noise trigger levels for other types of sensitive occupancies have been devised based on RING and industry knowledge. 2 Specified noise levels refer to noise from heavy or light rail transportation only (not ambient noise from other sources). 3 Assessment location is internal near to the centre of the most affected habitable room. 4 LASmax refers to the maximum noise level not exceeded for 95% of the rail pass-by events. 5 For schools, educational institutions, places of worship the lower value of the range is most applicable where low internal noise levels is expected. 6 The values for performing arts spaces may need to be reassessed to address the specific requirements of a venue. | All | Operation | NV038 |  |
|  | NV18 |  | Vibration Guideline Targets for Operation During operation, achieve the Guideline Targets (based on Table 1 in BS6472-1:2008) or background levels (whichever is higher) for vibration as follows: - Location; - VDV (m/s1.75). - Day 7:00am to 10:00pm; - Night 10:00pm to 7:00am. - Preferred Value; - Maximum Value; - Preferred Value; - Maximum Value. - Residences; - 0.20; - 0.40; - 0.10; - 0.20. - Offices, schools, educational institutions, places of worship; - 0.40; - 0.80; - 0.40; - 0.80. - Workshops; - 0.80; - 1.60; - 0.80; - 1.60. Notes 1 The Guideline Targets are non-mandatory; they are goals that should be sought to be achieved through the application of feasible and reasonable mitigation measures. 2 Compliance with these values implies no structural damage due to operation. | All | Operation | NV034 |  |
|  | New NVB* |  | - Construction Noise and Vibration Management Plan - Develop and implement a Construction Noise and Vibration Management Plan (“CNVMP”) in consultation with EPA Victoria and the relevant councils. The CNVMP must be informed by the modelling undertaken by the acoustic and vibration consultant in accordance with NV3 and must include (but not be limited to): - General - (1) identification of sensitive receivers along Melbourne Metro’s alignment; - (2) details of construction activities and an indicative schedule for construction works, including the identification of key noise and/or vibration generating construction activities (based on representative construction scenarios, including at ancillary facilities) that have the potential to generate noise and/or vibration impacts on surrounding sensitive receivers. - Airborne Noise - (3) identification of reasonable and practicable measures to be implemented to manage construction noise impacts having regard to: - (i) EPA Publication 1254 Noise Control Guidelines - (ii) the airborne construction noise guideline targets (internal) specified in NV5 - (iii) the management levels specified for airborne noise at residences during recommended standard hours in Part 4.1.1 of the NSW Interim Construction Noise Guidelines - (iv) the management levels specified for airborne noise at other sensitive land uses in Part 4.1.2 NSW Interim Construction Noise Guidelines - (v) the approach in Part 2.3 of the NSW Interim Construction Noise Guidelines when scheduling and planning for out of hours works (including unavoidable works) - (4) any management actions to be implemented if predicted noise levels exceed, for an extended period of time, the guideline targets specified in NV1 or NV5 (or any additional guideline targets specified in accordance with subclause 3 above); - (5) any measures to be implemented in accordance with the MMRA Residential Impact Mitigation Guidelines including (but not limited to) mitigation measures for out of hours works (including unavoidable works) where predicted noise levels exceed the noise levels specified in the Residential Impact Mitigation Guidelines. - Vibration: Structures - (6) Identification of any alternate vibration guideline targets to those specified in NV6, NV7 or NV8 deemed necessary and/or appropriate to protect the structural integrity of structures based on pre-construction condition surveys, undertaken in accordance with New CHA, GM4 and NV7 (or as otherwise required to assess the impact of vibration on structures along the alignment) - (7) identification of reasonable and feasible measures to be implemented to manage construction vibration impacts in accordance with the: - (i) vibration guideline targets for structures specified in, or otherwise determined in accordance with, NV6 - (ii) construction vibration limits for above and below ground utility assets determined in accordance with NV7 - (iii) vibration guideline targets for underground infrastructure specified in, or as otherwise determined in accordance with NV8 - (8) any management actions to be implemented if predicted vibration levels exceed, for an extended period of time, the guideline targets specified in NV6, NV7, or NV8, or otherwise determined in accordance with NVB*(6) - (9) specific heritage measures where relevant in accordance with CH2. - Vibration and Ground-borne Noise: Human Comfort - (10) identification of reasonable and practicable measures to be implemented to manage construction vibration and ground-borne noise impacts in accordance with the: - (i) vibration dose values for human comfort specified in NV9 (which may be expressed as peak particle velocity rates for the purposes of the CVNMP) - (ii) ground-borne (internal) noise guideline targets for amenity specified in NV11 - (11) any management actions to be implemented if predicted vibration or ground-borne noise levels exceed, for an extended period of time, the guideline targets identified in NV9 or NV11 - (12) any measures to be implemented in accordance with the Residential Impact Mitigation Guidelines including (but not limited to) mitigation measures for out of hours works (including unavoidable works) where ground-borne noise levels are predicted to exceed the ground-borne noise construction targets specified in the Residential Impact Mitigation Guidelines. - Vibration and Ground-borne Noise: Sensitive Equipment and Bio-resources - (13) identification of reasonable and practicable measures, to be determined following consultation with the Parkville Precinct Reference Group and RMIT, to be implemented to manage construction vibration and ground-borne noise impacts in accordance with the: - (i) vibration sensitive equipment guidelines specified in, or as otherwise determined in accordance with NV10 - (ii) bio-resource guideline targets specified in, or as otherwise determined in accordance with NV13 - (14) any management actions to be implemented if predicted vibration or ground-borne noise levels exceed, for an extended period of time, the guideline targets identified in NV10 or NV13 - Blasting - (15) if blasting is proposed, an assessment of the potential noise and vibration impacts associated with blasting activities, and the identification of measures to ensure compliance with Australian Standard AS2187.2-2006 as specified in NV12 - (16) any measures to be implemented in accordance with the Residential Impact Mitigation Guidelines - Community Consultation - (17) details of all community consultation measures to be implemented in accordance with NV4 and SC2 including: - (i) any precinct-specific community consultation measures; and - (ii) the establishment of measures concerning complaints management. - Haulage - (18) operational procedures and controls that minimise truck noise, including, but not limited to, consideration of the following: - (i) Where reasonable and practicable, limit heavy construction vehicle movements to Normal Working Hours (as defined by the EPA) providing this limitation does not include vehicles essential to maintaining construction operations; - (ii) Where practical, select different traffic routes to limit the amount of accelerating and braking, prioritise routes with existing heavy vehicle usage where possible, and avoid local roads (e.g. residential streets), particularly for 24-hour activities; - (iii) Install ‘no engine braking’ signs on designated routes; - (iv) Ensure trucks are fitted with mufflers that comply with the original equipment manufacturer specifications and relevant EPA in-service noise requirements; - (v) Enforce speed restrictions on all construction vehicles; - (vi) Complete regular maintenance checks of road surfaces and trucks; - (vii) Implement temporary changes to traffic light sequences on designated routes to minimise trucks starting and stopping at junctions; - (viii) Monitor construction vehicle driver behaviour; - (ix) Identify locations for trucks to idle pending arrival at construction sites; - (x) Minimise the need for trucks to reverse and require the use of broadband reverse alarms; - (xi) Address to the extent practicable noise from any truck wash required as vehicles leave construction sites (particularly at night). - Monitoring - (19) mechanisms to ensure effective monitoring of noise and vibration associated with construction in accordance with NV3, including: - (i) vibration and noise measurement methodologies for monitoring both baseline and construction levels, including details of the parameters to be obtained, the measurement equipment, parameters to be recorded, and relevant standards to be adhered to for the collection and analysis of data; - (ii) baseline and construction noise and vibration monitoring locations; - (iii) the most critical periods, whether determined separating distance or ground conditions, and the duration of monitoring periods; - (iv) specific measures, to be determined following consultation with relevant stakeholders, with respect to sensitive equipment and biological resources (which must, where practicable, include continuous monitoring during construction); - (iv) how the results of monitoring would be recorded, reported, and interpreted. |  | All | Construction |  | - Joint witness statement Item 6 and 25 - Evidence from S. McHugh - Joint witness statement Item 11 - Evidence from S,McHugh - Addressing RMIT submission in part - Joint witness statement Item 32 - Joint witness statement Item 15 |
| Electro Magnetic Interference (EMI) |  |  |  |  |  |  |  |  |
|  | EMI1 |  | - During detailed design: - undertake a project wide Electro Magnetic Interference (EMI) assessment for existing infrastructure, systems and equipment considering: - Baseline conditions - Stakeholder requirements - Manufacturer specifications of sensitive equipment - The electromagnetic emissions generated by the operation of any electrical or electronic equipment to be used during construction and operation of the project - Undertake baseline monitoring in accordance with sensitive equipment manufacturer environmental test requirements, where available. - Determine operational EMI limits in consultation with equipment owners having regard to equipment manufacturer environmental specifications where available and background EMI levels - If EMI limits are expected to be exceeded, as a result of the construction and/or operation of the project, design mitigation measures to minimise impact on sensitive equipment in accordance with relevant industry standards. |  | All with focus on 4 – Parkville station 5 – CBD North station | Design |  | Recommendation from University of Melbourne 15/09/16, and Joint witness statement Items 34, 35 & 36 |
|  | EMI2 |  | - Prior to construction and operation, prepare and implement an Electro Magnetic Compatibility (EMC) Management Plan that includes the following (but is not necessarily limited to): - Considers the electromagnetic emissions generated by the Works - Identifies sensitive receivers and the management measures - Includes a testing strategy to monitor performance of appropriate management measures - Outlines a program for regular auditing of electronic and electrical systems during the construction, testing and commissioning. - Outlines remedial action if EMI limits are not met systems during the construction, testing and commissioning. |  | All with focus on 4 – Parkville station 5 – CBD North station | Construction / Operation |  | Recommendation from University of Melbourne 15/09/16, and Joint witness statement Items 34, 35 & 36 |
| Aboriginal Cultural Heritage |  |  |  |  |  |  |  |  |
| Cultural Heritage – To avoid or minimise adverse effects on Aboriginal and historical cultural heritage values | AH1 |  | Comply with a Cultural Heritage Management Plan approved under the Aboriginal Heritage Act 2006 and prepared in accordance with the Aboriginal Heritage Regulations 2007. |  | All | Construction | AH001 to AH007 |  |
| Historical Cultural Heritage |  |  |  |  |  |  |  |  |
| Cultural Heritage – To avoid or minimise adverse effects on Aboriginal and historical cultural heritage values | CH1 |  | Design permanent and temporary works to avoid or minimise impacts on the cultural heritage values of heritage places. Consult, as required, with Heritage Victoria and/or the responsible authority (as applicable). |  | All | Detailed design | All |  |
|  | CH2 |  | To avoid or minimise impacts on the cultural heritage values of heritage places: Prepare and implement a Heritage Management Plan (HMP), which must identify the mitigation measures to be adopted to avoid or minimise impacts on the cultural heritage values of heritage places Perform works in accordance with the following noise and vibration and ground movement Environmental Performance Requirements as related to heritage places: New NVA, NV2, NV35, NV6, NV11, GM2, GM4, GM5, GM6 Undertake condition assessments of heritage places prior to commencement of construction where located within the identified vibration and ground settlement zones of sensitivity and monitor as per NV6, GM4 and GM5. Should damage occur to a building or structure on the Victorian Heritage Register or that is subject to a Heritage Overlay as a result of works, undertake rectification works in accordance with accepted conservation practice (with reference to the Australia ICOMOS Burra Charter 2013) and in consultation with Heritage Victoria, the land owner or the local Council, as applicable. |  | All | Prior to and during construction | HH03 | MMRA review to address questions about how management measures for heritage places will be identified. And - Graduate Union changes in part - Agreed with CoM 13/09/16 |
|  | CH3 |  | Prior to construction, undertake archival photographic recording in accordance with Heritage Victoria Technical Note: Photographic Recording for Heritage Places and Objects where heritage places are to be demolished or modified. |  | All | Prior to construction commencing | HH04, HH05, HH06, HH07, HH08, HH09, HH10, HH13, HH14, HH15, HH16, HH22, HH23, HH26, HH27, HH28, HH29, HH32 |  |
|  | CH4 |  | Prior to construction of main works or shafts that affect heritage structures or places, develop detailed methodology in accordance with Australia ICOMOS Burra Charter and in consultation with Heritage Victoria or the responsible authority (as applicable) where heritage fabric is required to be dismantled, stored and reconstructed. Work is to be documented and overseen by an appropriately qualified conservation practitioner. |  | 1 – Tunnels 3 – Arden station 6 – CBD South station 7 – Domain station | Prior to construction, reinstatement at an appropriate time during or after the main construction works | HH04, HH17, HH19, HH23, HH25, HH27, HH31 | Agreed with CoM 13/09/16 |
|  | CH5 |  | Prior to construction of main works or shafts that affect heritage structures or places, develop and implement appropriate protection measures for heritage places and objects including sculptures, memorials, monuments and associated heritage fabric where retained in proximity to works. This is to be done in consultation with Heritage Victoria or the responsible authority (as applicable). |  | All | Prior to construction commencing | HH04, HH05, HH18, HH25, HH26, HH29, HH31 | Agreed with CoM 13/09/16 |
|  | CH6 |  | In consultation with Heritage Victoria: Develop archaeological management plans to manage disturbance of archaeological sites and values affected by the project Undertake investigation in accordance with the Guidelines for Investigating Historical Archaeological Artefacts and Sites, Heritage Victoria 2014 (as amended or updated) and to the satisfaction of the Executive Director, Heritage Victoria. Develop and implement a protocol for managing previously unidentified historical archaeological sites discovered during project works. |  | All | Pre-construction | HH01, HH02 | Agreed with CoM 13/09/16 |
|  | CH7 |  | Develop and implement a heritage interpretation strategy as part of detailed design as a whole which seeks to explore historical and Aboriginal cultural heritage themes. This must include (but not be limited to) the exploration of opportunities for interpretation at Arden station (referencing the use of this land for railways workshops and sidings), and at CBD South station (referencing the Port Phillip Arcade and the early Port Phillip Club Hotel). |  | All | Design/ Construction | Specific risks include HH13 and HH23 but the opportunities apply across the project. | Agreed with CoM 13/09/16 |
|  | CH8 |  | Undertake all underground service works beneath or within heritage places or tree protection zones (TPZs) for trees as part of heritage places to avoid, minimise and mitigate impacts to the heritage fabric. |  | All | Construction | HH33, HH34 | Agreed with CoM 13/09/16 |
|  | CH9 |  | Ensure new development is responsive to heritage places in terms of height, massing, form, façade articulation and materials. |  | All | Detailed design | HH05, HH06, HH08, HH18, HH20, HH21, HH22, HH24, HH26, HH27, | Agreed with CoM 13/09/16 |
|  | CH10 |  | Ensure no direct impact on heritage buildings on the former Glueworks site in Kensington. |  | 2 – Western portal | Construction | HH12 | Agreed with CoM 13/09/16 |
|  | CH11 |  | Retain and protect Langford Street pumping station (part of proposed Moonee Ponds Creek and Infrastructure Precinct) as part of the design for the new substation. |  | 3 – Arden station | Detailed design | HH14 | Agreed with CoM 13/09/16 |
|  | CH12 |  | Replace removed Elm trees in Royal Parade as part of project delivery using appropriate species and re-establish the boulevard formation. Provide suitable soil conditions to facilitate the growth of new trees to reach the size of the existing mature trees in the boulevard. |  | 4 – Parkville station | Construction | HH15, HH16 | Agreed with CoM 13/09/16 |
|  | CH13 |  | In detailed design ensure the eastern Parkville station entry is set no less than 8-10 metres from the original Gatekeeper’s Cottage and an appropriate boundary treatment is retained or re-established for the heritage building. |  | 4 – Parkville station | Detailed design | HH18 | Agreed with CoM 13/09/16 |
|  | CH14 |  | In detailed design for the CBD South station, incorporate the Charles Bush sculpture into the design for the new building on the Port Phillip Arcade site, preferably in a prominent position on the Flinders Street façade. |  | 6 – CBD South station | Detailed design | HH23 | Agreed with CoM 13/09/16 |
|  | CH15 |  | In the event of temporary or permanent relocation of the Burke and Wills Monument from its current site is required, resolve the final location of the monument in consultation with the City of Melbourne prior to the commencement of construction. (See Environmental Performance Requirement CH4) |  | 6 – CBD South station | Detailed design | HH25 | Agreed with CoM 13/09/16 |
|  | CH16 |  | Integrate the bluestone pillar and cast iron fencing at the corner of Grattan Street and Royal Parade into the design for the station entry and surrounds in consultation with the University of Melbourne. |  | 4 – Parkville station | Detailed design | HH19 |  |
|  | CH17 |  | Replace removed trees as part of project delivery in accordance with relevant policy documents and to re-establish valued landscape character and in consultation with the City of Melbourne, the City of Port Phillip, the Shrine of Remembrance and Shrine Trustees and Heritage Victoria as applicable. Policy documents are as follows: Domain Parklands Conservation Management Plan 2016 and the Domain Parklands Masterplan (in preparation) Shrine of Remembrance Conservation Management Plan (Lovell Chen, 2010) or any future review and the Shrine of Remembrance Landscape Improvement Plan (rush Wright Associates, 2010) South African Soldiers Memorial Conservation Management Plan ( Context, 2016) |  | 1 – Tunnels 7 – Domain station | Construction | HH04, HH05, HH06, HH07, HH08, HH09, HO27, HH29, HH30 |  |
|  | CH18 |  | Review the siting and design of the eastern Domain station entry in detailed design to ensure it is as recessive as possible in this location and has only a limited presence on the edge of the Shrine of Rememberance Reserve. The design needs to allow for the maintenance of an appropriate setting to the Macpherson Robertson Memorial Fountain. |  | 7 – Domain station | Detailed design | HH26 | Agreed with CoM 13/09/16 |
|  | CH19 |  | In consultation with Heritage Victoria: Prior to dismantling the South African Soldiers Memorial, in consultation with City of Port Phillip develop interpretive material to display in the precinct until the monument is restored. For detailed design, in consultation with City of Port Phillip review the siting and design of the western Domain station entry in detailed design to ensure the South African Soldiers Memorial has an appropriate landscaped setting if relocated on this site. If no appropriate setting can be established, consider options for relocation of the memorial to an alternative site. |  | 7 – Domain station | Detailed design | HH27 | Agreed with CoM 13/09/16 |
|  | CH20 |  | the City of Melbourne, City of Port Phillip and/or the responsible authority In consultation with VicRoads, Heritage Victoria and/or relevant local councils, as applicable replace removed trees in St Kilda Road to re-establish the boulevard formation. |  | 7 – Domain station | Detailed design | HH28 | Agreed with CoM 13/09/16 |
|  | CH21 |  | Retain and protect the Cross Street Electrical Substation in situ within or abutting proposed construction site. |  | 9 – Western turnback | Construction | HH35 |  |
|  | CH22 |  | Ensure that, where impacted by project works, street fabric and infrastructure is conserved and/or accurately reconstructed. |  | All | Construction |  |  |
|  | New CHA* |  | Before tunnelling commences: Consider the construction noise and vibration modelling required by EPR NV3 and identify heritage places on the Victorian Heritage Register that may be vulnerable to degradation as a result of vibration from construction and identify appropriate mitigation measures to prevent damage to heritage places from vibration Conduct pre-construction condition surveys of heritage places identified in the modelling as potentially being vulnerable to degradation as a result of vibration, to record structural condition and structural integrity prior to the commencement of tunnelling Implement the identified mitigation measures to prevent damage to heritage places from vibration Conduct vibration monitoring at the heritage places that may be vulnerable to degradation to assess the actual vibration from construction works. If the vibration monitoring demonstrates the condition of heritage places may be degraded as a result of vibration, ground vibration must be reduced until the risk of vibration related degradation is assessed as acceptable. |  | All | Construction |  |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Historical Cultural Heritage’: NV2, NV35, NV6, NV7, NV11, GM2, GM4, GM5, GM6 |  |  |  |  |  |  |  |
| Arboriculture |  |  |  |  |  |  |  |  |
| Landscape, visual and recreational values – To avoid or minimise adverse effects on landscape, visual amenity and recreational values as far as practicable | AR1 |  | During detailed design, review potential tree impacts and provide for the maximum tree retention on both public and private land, also having regard to valuable habitat linkages or corridors where possible practicable. Prior to construction of main works and shafts, develop and implement a plan in consultation with the relevant local council that identifies all trees in the project area which covers: Trees to be removed or retained Condition and significance of the trees to be removed Options for temporary re-location of palms and reinstatement at their former location or another suitable location. - The plan should include a tree removal protocol that includes a process for MMRA approval of trees prior to removal. |  | All | Design/Pre-construction | AR001, AR002, AR014, AR003 | National Trust Suggestion 19/09/16 Review by MMRA |
|  | AR2 |  | Reinstate quality soils to sufficient volumes to support long-term viable growth of replacement trees. Ensure ongoing supply of water to tree root zones, especially during their establishment stage. Employ water sensitive urban design principles (WSUD) principles where possible. |  | All | Construction | AR001, AR002, AR003, AR014, |  |
|  | AR3 |  | Re-establish trees to replace loss of canopy cover and achieve canopy size equal to (or greater than) healthy, mature examples of the species in Melbourne. Consult with the City of Melbourne, the City of Port Phillip, the City of Stonnington, the Shrine of Remembrance and Shrine Trustees, University of Melbourne and Heritage Victoria as applicable. Policy documents that should be referenced to re-establish trees and valued landscape character include: - The City of Melbourne’s Tree Retention and Removal Policy and Urban Forest Strategy, South Yarra Urban Forest Precinct Plan, Central City Urban Forest Precinct Plan and Carlton Urban Forest Precinct Plan (excluding sections 8.2 and 8.3) The City of Port Phillip’s Community Amenity Local Law No. 1 and Greening Port Phillip – An Urban Forest Approach The City of Stonnington’s General Local Law 2008 (No 1) and City of Stonnington Street Tree Strategy Any associated precinct plans Specific policies of the Domain Parklands Conservation Management Plan for trees within Domain Parklands Shrine of Remembrance Conservation Management Plan (Lovell Chen, 2010) or any future review and the Shrine of Remembrance Landscape Improvement Plan (rush Wright Associates, 2010) South African Soldiers Memorial Conservation Management Plan (in preparation, Context, 2016) The preferred future character of the University of Melbourne, for trees in the grounds of the University of Melbourne. - The re-establishment of trees must also consider the contribution to creation of habitat corridors and linkages where possible. |  | All (except Western turnback) | Construction | AR001, AR002, AR003, AR014, | Agreed with CoM 13/09/16 and University of Melbourne Changes 15/9/16 |
|  | AR4 |  | Prior to construction commencing of main works or shafts in affected areas, prepare and implement Tree Protection Plans for each precinct in accordance with AS4970-2009 Protection of Trees on Development Sites, addressing the detailed design and construction methodology of the project. Within precincts 1, 4 and 7 a Tree Protection Plan must be developed for each heritage place as relevant to the satisfaction of Heritage Victoria or the responsible authority. |  | All (except Western Turnback) | Construction | AR001 to AR005, AR007 to AR012, AR014 |  |
|  | AR5 |  | For City of Melbourne trees that are to be retained and protected, a bank guarantee or bond of the trees value will be held against the approved Tree Protection Plan for the duration of the works in accordance with the city of Melbourne Tree Retention and Removal Policy. |  | All (except Eastern Portal and Western turnback) | Construction | AR004 to AR005, AR007 to AR012 |  |
| Landscape and Visual |  |  |  |  |  |  |  |  |
| Landscape, visual and recreational values – To avoid or minimise adverse effects on landscape, visual amenity and recreational values as far as practicable |  | LV1 | Develop and implement a plan for the design of permanent and temporary works in consultation with local councils and the Office of Victorian Government Architect to comply with the Melbourne Metro Urban Design Strategy. Avoid or minimise to the extent practicable, visual impacts on sensitive receptors and maintain broader landscape character values, particularly in relation to: Tunnels: Queen Victoria Gardens, Tom’s Block, Fawkner Park Western Portal: JJ Holland Park Parkville Station: University of Melbourne, Victorian Comprehensive Cancer Centre, Royal Melbourne Hospital, University Square CBD North Station: Royal Melbourne Institute of Technology, the State Library and State Library Forecourt CBD South Station: St Paul’s Cathedral, Federation Square, City Square and Flinders Street Station Domain Station: The Shrine of Remembrance, Shrine of Remembrance Reserve, Albert Road Reserve, Domain Parklands Eastern Portal: South Yarra Siding Reserve. Existing habitat corridors within and proximate to Moonee Ponds Creek, if the alternate substation site adjacent to the Moonee Ponds Creek is selected - Consult with University of Melbourne in relation to location and design of station entries on University land. |  | All | Construction/ Operation | LV001 to LV050 | Agreed with CoM 13/09/16 |
|  |  | LV2 | Develop and implement a plan in consultation with the Office of Victorian Government Architect, local councils and other land managers to comply with the Melbourne Metro Urban Design Strategy to re-establish and enhance where appropriate public open space, recreation reserves and other valued places disturbed by temporary works. The plan must include, but not be limited to a methodology and timeframe for storage, reinstatement or replacement of existing public art, monuments and public infrastructure such as poles (including banner poles), bins, and other street furniture such as wayfinding signage (including signage hubs) The plan should also include exploring opportunities for renewal of public spaces for the benefit of communities beyond resident groups, including visitors, business owners and commuters. The plan should include a timeframe for re-establishment of public open space, recreation reserves and other valued places disturbed by temporary works. |  | All | Design/ Construction | LV001 to LV050 | Agreed with CoM 13/09/16 |
|  |  | LV3 | Prior to construction, develop measures to minimise light spillage during construction to protect the amenity of adjacent neighbourhoods, parks and community facilities. Lighting for operation would be designed in accordance with council requirements and relevant standards. |  | All | Construction | LV001 to LV025 |  |
| Contaminated Land and Spoil Management |  |  |  |  |  |  |  |  |
| Hydrology, water quality and waste management: To protect waterways and waterway function and surface water and groundwater quality in accordance with statutory objectives, to identify and prevent potential adverse environmental effects resulting from the disturbance of contaminated or acid-forming material and to manage excavation spoil and other waste in accordance with relevant best practice principles | C1 |  | Prior to construction of main works or shafts, prepare and implement a Spoil Management Plan (SMP) in accordance with MMRA’s Spoil Management Strategy and relevant regulations, standards and best practice guidance. The SMP shall be developed in consultation with and to the satisfaction of the EPA. The SMP will include but is not limited to the following: Applicable regulatory requirements Identifying nature and extent of spoil (clean fill and contaminated spoil) across all precincts Roles and responsibilities Identification of management measures for handling and transport of spoil for the protection of health and the environment Identification, design and development of specific environmental management plans for temporary stockpile areas Identifying suitable sites for re-use, management or disposal of any spoil Monitoring and reporting requirements Identifying locations and extent of any prescribed industrial waste (PIW) and characterising PIW spoil prior to excavation Identifying suitable sites for disposal of any PIW. The SMP shall include sub-plans as appropriate, including but not limited to an Acid Sulfate Soil and Rock (ASS/ASR) Management Sub-Plan (Refer to C2). |  | All | Construction | CL001, CL016, CL021 to CL026, CL043, CL048, CL049 |  |
|  | C2 |  | Prepare and implement an Acid Sulfate Soil and Rock (ASS/ASR) Management Sub-Plan prior to construction of the project as a sub-plan of an overarching SMP in accordance with the regulations, standards and best practice guidance and to the satisfaction of EPA. This sub-plan will include the general requirements of the SMP and also: Identify locations and extent of any potential ASS/ASR Characterise ASS/ASR spoil prior to excavation Identify and implement measures to prevent oxidation of ASS/ASR wherever possible Identify suitable sites for re-use, management or disposal of any ASS/ASR. |  | All | Construction | CL002, CL005, CL006, CL018, CL044, CL046 |  |
|  | C3 |  | Prior to construction of main works or shafts, prepare a Remedial Management Plan (RMP) undertake a remedial options assessment (ROA) for contaminated land and groundwater. The assessment RMP must: Consider the outcomes of further investigations including the appropriate groundwater investigations and modelling required in GW1, GW2, GW3 and GW5 Interpret groundwater permeation and VOC results Present and take account of the outcomes of risk assessments If required, identify remedial options to be implemented for contaminated land and groundwater in accordance with relevant regulations, standards and best practice guidance and to the satisfaction of EPA. If required, as an outcome of the ROARMP, prepare and implemented a remedial action plan and integrate the remediation approach into the design of the Project in accordance with relevant regulations, standards and best practice guidance and to the satisfaction of EPA. |  | All | Construction/ Operation | CL008, CL010, CL013, CL027, CL030, CL033, CL036, CL039, CL040, CL050, CL052, CL054, CL056, CL058 |  |
|  | C4 |  | Prior to construction of main works or shafts commencing, prepare and implement a health, safety and environmental plan for the management of hazardous substances. The plan must include but not be limited to: Consideration of the risks associated with exposure to hazardous substances for employees, visitors and general public The identification of methods to control such exposure in accordance with relevant regulations, standards and best practice guidance and to the satisfaction of WorkSafe and EPA Method statements detailing monitoring and reporting. |  | All | Construction | CL015, CL042, CL059 |  |
| Groundwater |  |  |  |  |  |  |  |  |
| Hydrology, water quality and waste management: To protect waterways and waterway function and surface water and groundwater quality in accordance with statutory objectives, to identify and prevent potential adverse environmental effects resulting from the disturbance of contaminated or acid-forming material and to manage excavation spoil and other waste in accordance with relevant best practice principles | GW1 |  | Design the tunnel and underground structures so that they minimise groundwater drawdown during construction and operation to minimise impacts on groundwater dependent values, ground movement and contamination plume migration. |  | All | Design | GW001 to GW059 |  |
|  | GW2 |  | Develop a groundwater model in a process that involves ongoing referral to the Independent Environmental Auditor consistent with the Australian Groundwater Modelling Guidelines (Barnett et al, 2012). Apply the model for the detailed design phase to predict impacts associated with any changes to construction techniques or operational design features proposed during detailed design, and reconfirm that the Environmental Performance Requirements and mitigation measures are sufficient to mitigate impacts from changes in groundwater levels, flow and quality. The groundwater model should be updated to address comprehensively; transient calibration, aquifer specific storage parameter values and their justification, prediction of cumulative impacts during construction and uncertainty assessments. Undertake monitoring during construction to ensure that predictions are accurate and mitigation measures are appropriate. |  | All | Design/construction | GW001 to GW059 |  |
|  | GW3 |  | Develop and implement a Groundwater Management Plan (GMP) detailing groundwater management approaches to address the predicted impacts to groundwater dependent values during construction. The GMP must be based on the detailed design phase groundwater model, and should include the following details: Approach to collection, treatment and disposal of groundwater collected during construction in accordance with the MMRA Groundwater Disposal Strategy Identifying and if necessary, specifying mitigation measures to protect groundwater dependent vegetation during periods of drawdown An approach identified in consultation with the EPA so that contaminant migration cause no significant impacts on beneficial uses and vapour intrusion into underground structures, and establish appropriate monitoring networks to confirm effectiveness of approach Methods for minimising drawdown in areas of known PASS and establishing appropriate monitoring networks to confirm effectiveness of approach |  | All | Design/Construction | GW001 to GW059 |  |
|  |  |  | - Methods for minimising drawdown at any existing recharge bores, and establishing appropriate monitoring networks to confirm effectiveness of mitigation - Groundwater drawdown trigger levels for groundwater dependant values at which additional mitigation measures must be adopted - Design, operation and management of groundwater injection borefields - Contingency measures if impacts occur at existing active groundwater bores and surface water bodies - Contingency measures should unexpected groundwater conditions be encountered. - The GMP must satisfy the EPA and relevant water authorities that groundwater dependent values would be protected. The GMP should also address MMRA’s sustainability requirements where appropriate. |  |  |  |  |  |
|  | GW4 |  | Use the Groundwater Disposal Strategy and GMP to obtain a Trade Waste Agreement with the relevant Water Retailers for groundwater disposal. |  | All | Construction/ Operation | GW055, GW056 |  |
|  | GW5 |  | - Develop and implement a groundwater monitoring plan as part of the GMP that details sufficient monitoring of drawdown to verify that no significant impacts occur from potential: Contaminant migration on the beneficial uses of groundwater at third party properties caused by drawdown and vapour intrusion to underground structures Activation of PASS and groundwater acidification Reduction in access to water for bore owners in the area around the project Reduction in access to groundwater for trees– particularly in the Tunnels precinct between CBD South and Domain stations, and the CBD South station and eastern portal precincts Change in groundwater levels in any existing recharge bores that may be present in the area around the project. |  | All | Construction/Operation | GW017, GW019, GW021, GW025, GW027, GW028, GW029, GW030, GW033, GW038, GW039, GW045, GW046 |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Groundwater’: GM1, AE3, AE6 |  |  |  |  |  |  |  |
| Ground Movement and Land Stability |  |  |  |  |  |  |  |  |
| Land Stability – To avoid or minimise adverse effects on land stability that might arise directly or indirectly from project works | GM1 |  | Develop and maintain geological and groundwater models (as per GW2) which: Use monitored ground movement and ground water levels prior to construction to identify pre-existing movement Inform tunnel design and the construction techniques to be applied for the various geological and groundwater conditions Assess potential drawdown and identify trigger levels for implementing additional mitigation measures to minimise potential primary consolidation settlement Assess potential ground movement effects from excavation and identify trigger levels for implementing additional mitigation measures to minimise potential ground movement effects. |  | All | Design/ Construction | GM001 to GM025 |  |
|  | GM2 |  | Design and construct the permanent structures and temporary works to limit ground movements to within appropriate acceptability criteria (to be determined in consultation with relevant stakeholders, local councils and land managers) for vertical, horizontal, and angular deformation as appropriate for project activities during the construction and operational phase. |  | All | Design/ Construction/ Operation | GM001 to GM025 | Agreed with CoM 13/09/16 |
|  | GM3 |  | Develop and implement a ground movement plan for construction and operational phases of the project that: Addresses the location of structures/assets which may be susceptible to damage by ground movement resulting from Melbourne Metro works, having particular regard to places listed on the Victorian Heritage Register Identifies appropriate ground movement impact acceptability criteria for buildings, utilities, trains, trams and pavement after consultation with the various stakeholders Identifies mitigation measures to ensure acceptability criteria can be met Identifies techniques for limiting settlement of buildings and protecting buildings from damage Addresses additional measures to be adopted if acceptability criteria are not met such as reinstatement of any property damage Establishes monitoring ground movement monitoring requirements for the area surrounding proposed Melbourne Metro works and at the location of various structures/assets to measure consistency with the predicted model Consult with land and assets owners that could potentially be affected and where mitigation measures would be required. |  | All | Construction/ Operation | GM001 to GM025 | Agreed with CoM 13/09/16 |
|  | GM4 |  | - Conduct pre-construction condition surveys for the assets predicted to be affected by ground movement. - Develop and maintain a data base of as-built and pre-construction condition information for each potentially affected structure identified as being in an area susceptible to damage (GM3) and where a property owner has requested an assessment, specifically including: Identification of structures/assets which may be susceptible to damage resulting from ground movement resulting from Melbourne Metro works Results of condition surveys of structures, pavements, significant utilities and parklands to establish baseline conditions and potential vulnerabilities Records of consultation with landowners in relation to the condition surveys. Post-construction stage condition surveys conducted, where required, to ascertain if any damage has been caused as a result of Melbourne Metro. Share pre- and post-condition assessments and records of consultation with the property owner proactively. Ensure all stakeholder engagement activities are undertaken within the framework of the Community and Stakeholder Engagement Management Plan |  | All | Construction | GM001 to GM025 | - Updating the name of the plan as per SC3 |
|  | GM5 |  | Adopt construction techniques for Melbourne Metro to limit ground movement to within appropriate acceptability criteria (to be determined in consultation with relevant stakeholders). |  | All | Construction | GM001 to GM025 |  |
|  | GM6 |  | For properties and assets affected by ground movement, undertake any required repair works. |  | All | Construction | GM001 to GM025 |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Ground Movement and Land Stability’: GW1, GW3, CH2 |  |  |  |  |  |  |  |
| Surface Water |  |  |  |  |  |  |  |  |
| Hydrology, water quality and waste management: To protect waterways and waterway function and surface water and groundwater quality in accordance with statutory objectives, to identify and prevent potential adverse environmental effects resulting from the disturbance of contaminated or acid-forming material and to manage excavation spoil and other waste in accordance with relevant best practice principles | SW1 |  | For all precincts (with the exception of the western turnback) design permanent and temporary works and, if necessary, develop and implement emergency flood management measures for the tunnels, tunnel portals, access shafts, station entrances and Arden electrical substation to provide appropriate protection against floodwaters and overland stormwater flows. This would be informed by a flood immunity risk assessment that considers a range of events, and to the requirements and satisfaction of Melbourne Water and/or the relevant council. the responsible authority. |  | All (except western turnback) | Construction/ Operation | SW001 to SW003, SW005, SW007, SW009, SW010, SW011, SW014, SW015, SW018, SW019, SW022, SW024, SW025, SW027, SW029, SW030 to SW032 |  |
|  | SW2 |  | For all precincts: Maintain existing flood plain storage capacity potentially impacted by the project, to the requirements and satisfaction of the responsible waterway management authority Permanent and associated temporary construction works must not increase flood levels that result in an additional flood risk to the requirements and satisfaction of the responsible waterway management authority Ensure permanent and associated temporary works do not increase flow velocities that would potentially affect the stability of property, structures or assets, and/or result in erosion during operation or construction, to the requirements and satisfaction of the responsible waterway management authority Undertake modelling of the design of permanent and temporary works to demonstrate the resultant flood levels and risk profile to the satisfaction of the responsible waterway management authority Ensure that the stormwater design associated with the project is undertaken to the requirements and satisfaction of the responsible waterway management authority Adopt WSUD and integrated water management principles, as required through the Melbourne Metro Urban Design Strategy. |  | All | Construction/ Operation | SW004, SW006, SW008, SW012, SW013, SW016, SW017, SW020, SW021, SW023, SW026, SW028, SW033, SW034 |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Surface Water’: AE6 |  |  |  |  |  |  |  |
| Aquatic ecology and river health |  |  |  |  |  |  |  |  |
| Hydrology, water quality and waste management: To protect waterways and waterway function and surface water and groundwater quality in accordance with statutory objectives, to identify and prevent potential adverse environmental effects resulting from the disturbance of contaminated or acid-forming material and to manage excavation spoil and other waste in accordance with relevant best practice principles. Biodiversity – To avoid or minimise adverse effects on native terrestrial and aquatic flora and fauna, in the context of the project’s components and urban setting | AE1 |  | Fully integrate the stormwater treatment system into the design of Melbourne Metro (all precincts) for construction [all precincts] to ensure that stormwater entering a receiving water body complies with SEPP (Waters of Victoria). The best practice performance objectives for achieving compliance with SEPP (Waters of Victoria) during the construction phase are described below: Pollutant type; Receiving water objective; Current best practice performance objective(1). Suspended solids; Comply with SEPP; Effective treatment of 90% of daily run-off events (e.g. <4 months ARI). Effective treatment equates to a 50 percentile suspended solids concentration of 50 mg/L. This can be achieved by installing a sediment pond(s) to remove 95% of sediment down to 125 µm for a 1 year ARI.. Litter; Comply with SEPP; Prevent litter from entering the stormwater system.. Other pollutants; Comply with SEPP; Limit the application, generation and migration of toxic substances to the maximum extent practicable.. Notes 1 Best practice performance objectives are based on the Best Practice Environmental Management Guidelines for Urban Stormwater – CSIRO. | All | Construction | AE001, AE007, AE008 |  |
|  | AE2 |  | Best practice sedimentation and pollution control measures must be applied to protect waterways in accordance with Best Practice Environmental Management: Environmental Guidelines for Major Construction Sites – EPA publication 480 (1996) and in accordance with an approved construction environmental management plan. Measures should include: vehicle wheel wash and rumble bars at worksite egress points, appropriate placement of material stockpiles and chemical storages, covered loads, street sweeping and water quality monitoring, where required. |  | All | Construction | AE001, AE007, AE008 |  |
|  | AE3 |  | During construction, discharge tunnel, station box and portal construction water to sewer. Where groundwater interception during construction is predicted to occur, dewatering is to be managed so that groundwater is not released to stormwater or sensitive surface water bodies (refer to Environmental Performance Requirement GW4). |  | All (except Western turnback) | Construction | AE002 |  |
|  | AE4 |  | Where ground treatment works are required in waterways, design and implement methods that prevent discharge of sediments into the water column. |  | 1 – Tunnels | Construction | AE003 |  |
|  | AE5 |  | Design the Arden electrical substation (as per SW1) to provide appropriate protection against floodwaters during operation, to prevent the release of contaminants to Moonee Ponds Creek. |  | 3 – Arden station | Design/ Operation | AE012 |  |
|  | AE6 |  | During operation, discharge tunnel drainage water to sewer, unless otherwise agreed by EPA and Melbourne Water. Where groundwater interception during operation is predicted to occur, disposal is to be managed so that contaminated water is not released to stormwater or sensitive surface water bodies (refer to Environmental Performance Requirement GW4). |  | 1 – Tunnels | Operation | AE010 |  |
|  | AE7 |  | Fully integrate the stormwater treatment system into the design of the western portal and eastern all precincts portal to ensure that stormwater entering a receiving water body complies with SEPP (Waters of Victoria). The best practice performance objectives for achieving compliance with SEPP (Waters of Victoria) during the operations phase are described below: Pollutant type​; Receiving water objective​; Current best practice performance objective(1)​. Suspended solids (SS); Comply with SEPP (not to exceed the 90th percentile of 80 mg/L) (2)​; 80% retention of the typical urban annual load​. Total phosphorus (TP); Comply with SEPP (base flow concentration not to exceed 0.08 mg/L) (3); 45% retention of the typical urban annual load​. Total nitrogen (TN); Comply with SEPP (base flow concentration not to exceed 0.9 mg/L) (3); 45% retention of the typical urban annual load​. Litter​; Comply with SEPP (No litter in waterways) (2); 70% reduction of typical urban annual load (4)​. Flows​; Maintain flows at pre-urbanisation levels​; Maintain discharges for the 1.5 year ARI at pre-development levels​. Notes 1 Best practice performance objectives are based on the Best Practice Environmental Management Guidelines for Urban Stormwater – CSIRO. 2 An example using SEPP (Waters of Victoria), general surface waters segment. 3 SEPP Schedule F7 – Yarra Catchment – urban waterways for the Yarra River main stream. 4 Litter is defined as anthropogenic material larger than five millimetres. Sedimentation and pollution control measures must be applied to protect waterways and habitat areas such as periphery surrounding Moonee Ponds Creek in accordance with industry best practice. This shall include water quality monitoring, where required. | 2 – Western portal 8 – Eastern portalAll | Operation | AE009 |  |
| Terrestrial flora and fauna |  |  |  |  |  |  |  |  |
| Biodiversity – To avoid or minimise adverse effects on native terrestrial and aquatic flora and fauna, in the context of the project’s components and urban setting | FF1 |  | Where ‘unavoidable’ native vegetation (as defined under relevant policy) needs to be removed, meet the requirements of the Permitted Clearing of Native Vegetation – Biodiversity Assessment Guidelines. |  | All | Construction | TE005, TE006 |  |
|  | FF2 |  | Develop and implement measures to avoid the spread or introduction of weeds and pathogens during construction, including vehicle hygiene. |  | All | Construction | TE001, TE002, TE005, TE006 |  |
|  | FF3 |  | Prior to site clearance for construction, all vegetation being removed is to be inspected by a suitably experienced and qualified environmental officer for habitat features and fauna occupancy. Where non-listed species (native and exotic) are encountered, any individuals will be encouraged to leave the tree or vegetation. Where nests/young are encountered, they will be relocated to a similar tree (or habitat) in close proximity. Prior to site clearance for construction, develop a translocation plan for the management of listed fauna species if encountered. |  | All | Construction | TE001 to TE006 |  |
|  | Also refer to the following Environmental Performance Requirements for ‘Terrestrial Flora and Fauna’: AR3, AE1, AE2, AE3, AE4, AE5, AE6, AE7, AE8, NV4 |  |  |  |  |  |  |  |

* Numbers of new EPRs to be assigned when finalised